The shell works as follows:

Every vertical blank each machine reads the joystick and sends the value

to the other machine. It presumably will also receive a joystick value from

the other machine. The value it just sent gets stuck into a FIFO, and an entry

is taken out of the FIFO and matched with the joystick value just received.

This joystick pair gets stuck into another FIFO, called the DF (for Dave Fifo).

At the top of the main game loop a joystick pair is taken out of the DF and the

game's joystick values are updated.

The VBL interrupts are kept in sync with syncotron. This means as serial

data comes in, the time it arrived relative to vertical blank is examined. If

it arrived too late that means the other machine is slower that we are and we

have to go into interlace to slow ourselves down. If it arrives too early,

that means we're too slow and the other machine will go into interlace mode

to slow it down. Each frame 3 bytes are sent. There are 16 bits of data and

8 bits of checksum.

One bit of the data is used for maintaining the DF fifo properly. There are

15 bits left over. Super Street Fighter 2 uses 12 bits to send 3 or 6 button

joystick information. There is also a bit that tells whether the joystick is

3 or 6 button. Certain codes are not allowed. A value of 3 being sent, which

in the case of SSF2 would mean up and down both being set, has some meaning

for the information passing mechanism. If you use this value the system will

break down. The shell joystick read routine matches the SSF2 read routine,

and returns a value like the following:

; [MSB]  f... tzyx  sacb rldu   [LSB]

The f bit is the DF control bit, and the t bit tells whether it is a 6 or

3 button controller. In this mapping, we just want to avoid sending a byte

with a value of 3 in the bottom 8 bits.

The shell is ideal for games that complete their main game loop in 1

frame, and only occasionally take longer. Vertical blanks should never be

disabled. If a game frequently takes longer than 1 frame then the shell

will have to be modified.

The DF bit works as follows: Every vertical blank the joystick

is read and data is passed to the other machine (and we receive data from

the other machine). This means that every vertical blank we get a valid

joystick pair. We stuff these pairs into the DF, and the main game loop

will take them out. What happens if for some reason the main game loop takes

longer than 1 frame to complete? It will not have processed a joystick pair,

the pair will have stacked up in the DF. If this happens repeatedly, more

and more values will be stuck in the DF, and this will introduce a new

latency on top of the latency caused by the modem and phone line delays.

What we need is a mechanism to clean out the DF safely. The way we do it

is to have both machines agree to throw values away. The DF bit is used

so that both sides will know when it is safe to take some action to

correct the DF. Each side checks the depth of the DF, and if there is more

than 1 element in it, a 1 bit is sent for the DF bit.

Both sides get the next joystick pair at the top of the main game loop.

If both DF bits are set to 1, then both sides have an extra joystick

element that we can get rid of. Both sides throw away the first element

in the DF and get the next one. Then for safety reasons, both sides hold

off doing any corrective action on the DF for 8 frames.

It is necessary to wait these 8 frames because it will take some amount

of time for both sides to know that the corrective action either worked,

or additional action must be taken. Typical latency across the phone lines

is 4 frames. Sometimes it is as many as 6. Assuming a latency of 4, if

both sides have extra DF elements they will send 1 for the DF bit, and

four frames later the joystick pair will percolate down to the main game

loop. Both sides will throw away an element in the DF, take the next one,

and continue on with the main game loop. The next game loop the DF will

be OK and won't contain any extra elements, but when we get the joystick

pair we will still see that both DF bits are 1. In fact we will get 1's

for both DF bits for each frame of latency that we are operating at. If

we were to blindly take corrective action (as in throwing away DF entries)

we would throw away too much--in fact more than there is in the DF. So the

solution is to just take one corrective action, then to hold off doing

anything for 8 frames. 8 is guaranteed to be larger than any possible

latency.

In Super Street Fighter II, there are times where we go away from the

main game loop for as many as 8 or 9 entire frames, drawing the background

and setting up the display. During this, interrupts are still enabled,

the joystick is still being read, and values are passed back and forth

between the two machines, and joystick pairs are being stuffed into the

DF. When we finally come back to main loop, we see that we've got too

many elements and we start throwing them away at the rate of 1 every 8

frames. This means it takes about a second to work our way down to the

bottom. This is perfectly ok in SSF2 because due to the nature of the

game you are unable to control the characters for several seconds anyway

after these big blits. The game is saying stuff like "Get ready, go!"

The nature of other games may be different. If you cannot afford to wait

about a second before cleaning up the DF, some new method will have to

be used.

One possible solution would be to use more than 1 bit. We have 4 spare

bits (after the 12 needed to represent a 6 button joystick). We can use

these to inform the other machine of the depth of our DF. Once we know

how deep both sides are, we can take one action that fixes everything,

then do the usual hold off for 8 frames before doing anything again.

There is another problem that has to be addressed. Sometimes one

side takes more than 1 frame to complete its main game loop, but on the

other it completed its main game loop in 1 frame. This means that one side

has an extra DF element. If this happens repeatedly on one side, the player

on that side will suffer from greater and greater latency, and the game

will appear more and more unresponsive.

The solution the shell uses is to detect when one side is behind. The

case would be where there is a 1 DF bit for one side, and a zero for the

other. The one with the zero must wait a frame for the other machine

to process the joystick pair and catch up. Again, after this corrective

action, both sides hold off for 8 frames before doing anything.

If the game to be patched frequently requires more than one game

frame to complete its main game loop, then the shell as it stands won't

work. The syncotron mechanism is ideal for games that complete their main

game loop in one game frame, and it can handle cases where occasionally

one or both sides take longer. It may be possible to devise a modified

version that will make the assumption that both sides take an equal

number of frames to complete the main game loop, which will actually

be the case most of the time. When one side gets ahead of the other or

when both sides get behind in processing DF elements, we can take some

action. Hopefully this won't happen too often.

Most of the shell handles maintaining sync, passing joystick values

back and forth, detecting errors, and recovering from errors. The actual

patches necessary to install the shell into a game will be pretty small

compared to all this.

The mechanism for getting a stream of joystick pairs works by being

the first thing called at vertical blank interrupt. The vector at $78

will be changed according to what state the mechanism is in. When we

first want to get both sides in sync, we install the establishsyncvbl

routine into vector $78--and vertical interrupts should be enabled. This

vbl will install other vbl's, handle everything about getting the two

machines in sync, then when all is ok will install the runvbl into

vector $78. The runvbl handles all the IO and syncotron. It fetches

data from the modem, determines when it arrived relative to vertical

blank, and takes corrective action by turning on or off interlace if

any is needed. It creates the joystick pairs and stuffs them into the

DF. When all its work is done, it finally calls the game's own vertical

blank handler.

The hope is that the runvbl can complete its necessary tasks fast

enough so that it won't cause any problems for the real handler. If

timing is critical, some other solution must be devised.

If the runvbl detects an error of some kind, it basically throws up

its hands and starts over from scratch, and both sides will go back

into trying to establish sync. When this happens the game will just

pause for a second or more. During all this time we never return to

mainline code, and instead we stay in the various vertical blank

interrupt handlers. We don't want to pass any down to the game's own

handler.

---  Describe dev. modem

---  Describe sharing of resources between debugger and patch

---  Describe dev. tools

---  Describe patch mechanism

---  Describe patch mechanism for big carts