The xband modem contains 64K of battery backed RAM (in the rest of this

document, RAM always refers to this) and 512K of ROM (ROM always refers to

this). These both can appear within the 4 megabyte address space of the

Genesis. The CARTRIDGE can also appear in this space. The xband modem contains

the FRED chip and this is responsible for all the memory mapping functions

of the modem.